| Purpose: | To control high-risk work through formal authorisation, verification of precautions and coordinated closure of each permit. |
| --- | --- |
| Scope: | This procedure applies to Company and contractor work requiring a permit at projects, workplaces and facilities under Company control. |
| Duties & Responsibilities | (1) Project Manager (PM): Establishes the project permit system and provides resources. (2) Safety and Health Officer (SHO): Reviews permit requirements and monitors effectiveness. (3) Site Safety Supervisor (SSS): Coordinates permits and verifies site conditions. (4) Safety Coordinator (SC): Maintains the permit register and assists inspections. (5) Permit Issuer: Authorises work after confirming required precautions. (6) Performing Authority: Accepts the permit and controls the work crew. (7) Workers: Follow permit conditions and stop work when conditions change. |
| ISO Reference: | MS ISO 45001:2018 Clause 8 |

| Revision History |  |  |  |
| --- | --- | --- | --- |
| Revision No. | Revision Date | Originator | Description of Changes |
| 0 | 01 Sept 2026 | ISO Manager | Initial issue for implementation of MS ISO 45001:2018 Clause 8 |

| Abbreviation & Definition |  |
| --- | --- |
| JSA | Job Safety Analysis |
| OHSMS | Occupational Health and Safety Management System |
| PIC | Person in Charge |
| PM | Project Manager |
| PPE | Personal Protective Equipment |
| PTW | Permit to Work |
| SC | Safety Coordinator |
| SHO | Safety and Health Officer |
| SSS | Site Safety Supervisor |

| Step | Activity | PIC | Reference |
| --- | --- | --- | --- |
| 1 | Permit System 1.1 Application A PTW shall be used for work identified by legal, client, project or Company requirements as requiring formal authorisation. 1.2 Permit Types Applicable permits may include hot work, confined space, excavation, work at height, lifting, electrical isolation, energisation and other high-risk work. 1.3 Authorisation The PM shall appoint competent Permit Issuers and define their authority and permit limits. | PM / SHO | Project safety plan; Appointment records |
| 2 | Permit Request 2.1 Submission The Performing Authority shall request the permit before work starts and provide the work scope, location, duration, personnel and equipment details. 2.2 Supporting Documents The request shall include the approved method statement, JSA, isolation plan, drawings, competency evidence and certificates where applicable. 2.3 Coordination Simultaneous work and interfaces with other contractors, systems and operational areas shall be identified. | PM / SHO / PS / SSS | JSA; Method statement; PTW form |
| 3 | Site Verification 3.1 Inspection The Permit Issuer and Performing Authority shall inspect the work area and confirm that specified controls are in place. 3.2 Precautions Required isolation, testing, barricades, ventilation, access, rescue arrangements, fire protection and PPE shall be verified as applicable. 3.3 Acceptance The Performing Authority shall accept the permit conditions and brief the work crew before work begins. | PM / SHO / PS / SSS | PTW checklist; Isolation record |
| 4 | Permit Issue and Display 4.1 Issue The Permit Issuer shall sign and issue the PTW only when the work can proceed safely. 4.2 Validity The permit shall state its validity period, work boundaries, precautions and suspension conditions. 4.3 Display The current permit shall be displayed or readily available at the work location. Unauthorised alteration is prohibited. | PM / SHO / PS / SSS | Approved PTW; PTW register |
| 5 | Work Monitoring 5.1 Control The Performing Authority shall remain responsible for compliance with the permit and control of the work crew. 5.2 Inspection The SSS or SC shall monitor the work and record deviations requiring correction. 5.3 Change Work shall stop and the permit shall be suspended when conditions, scope, personnel, isolation or simultaneous activities change materially. 5.4 Shift Handover A permit continuing across a shift shall be formally handed over or reissued according to project rules. | PM / SHO / PS / SSS | Site inspection record; Shift handover record |
| 6 | Suspension, Extension and Closure 6.1 Suspension Suspended work shall be left in a safe condition and the permit status shall be recorded. 6.2 Extension A permit may be extended only after the work area and controls are reverified by authorised persons. 6.3 Completion The Performing Authority shall confirm that work is complete, persons and tools are clear and the area is safe. 6.4 Closure The Permit Issuer shall inspect the area, cancel the permit and authorise restoration or return to service where applicable. | PM / SHO / PS / SSS | Closed PTW; Restoration record |
| 7 | Records and Review 7.1 Records Issued, suspended, extended, cancelled and closed permits shall be entered in the PTW register and retained. 7.2 Review The SHO shall review permit findings, suspensions, violations and incidents to improve the system. 7.3 Audit Periodic checks shall verify permit quality, authorisation, workplace compliance and closure. | SHO / SSS / SC | PTW register; OHSMS-09S; OHSMS-10W |

| Verified By: Name: Eng Choon Leong Appointment: Executive Director Date: 01 Sept 2026 | Approved By: Name: Amos Haw Chee Seng Appointment: Managing Director Date: 01 Sept 2026 |
| --- | --- |

End